# Primary Boot

The Kria K26 SoM uses A/B boot concept, uses 2 image cointaining FSBL, PMU, ATF, U-Boot, if both are avaible then the Image Selector loads image A, if image A is not bootable (because we’re modifying it) then Image Selector loads image B and so on.

If both images are not bootlable (this doesn’t happen) then the Image Selector starts the Image Recovery.

Both Images are stored in the SoM QSPI memory, this is the map used for Kria SoMs:

| Offset (hex) | Size (hex) | Sector Access | Description | Linux mtd |
| --- | --- | --- | --- | --- |
| 0x00000000 | 0x80000 | R | Image Selector Application | 0 |
| 0x00080000 | 0x80000 | R | Image Selector Application Golden | 1 |
| 0x00100000 | 0x20000 | R/W | Image Selector registers | 2 |
| 0x00120000 | 0x20000 | R/W | Image Selector register backup | 3 |
| 0x00140000 | 0xC0000 | - | Reserved | 4 |
| 0x00200000 | 0x300000 | R/W | Image A ( BOOT.BIN containing FSBL, PMU, ATF, U-Boot) | 5 |
| 0x00500000 | 0xA00000 | R/W | Space for larger images (PL - 8MB, RPU - 2MB) | 5 |
| 0x00F00000 | 0x80000 | R | ImgSel Image A Catch (ImgSel application) | 6 |
| 0x00F80000 | 0x300000 | R/W | Image B ( BOOT.BIN containing FSBL, PMU, ATF, U-Boot) | 7 |
| 0x01280000 | 0xA00000 | R/W | Space for larger images (PL - 8MB, RPU - 2MB) | 7 |
| 0x01C80000 | 0x80000 | R | ImgSel Image B Catch (ImgSel application) | 8 |
| 0x01D00000 | 0x100000 | - | reserved | 9 |
| 0x01E00000 | 0x200000 | R | Image Recovery Application | 10 |
| 0x02000000 | 0x200000 | R | Image Recovery Application backup | 11 |
| 0x02200000 | 0x20000 | R/W | U-Boot storage variables | 12 |
| 0x02220000 | 0x20000 | R/W | U-Boot storage variables backup | 13 |
| 0x02240000 | 0x40000 | R | QSPI Image Version Information* | 14 |
| 0x02280000 | 0x20000 | R/W | Secure OS storage | 15 |
| 0x022A0000 | 0x1d60000 | R/W | End Address (User) | 16 |

Fonti: https://xilinx.github.io/kria-apps-docs/bootfw/build/html/docs/bootfw_overview.html#primary-boot-device

| 0x00100000 | 0x20000 |
| --- | --- |

Image Selector Registers¶

| Offset | Register | Description | Default Value for SOM Manufacturing Image |
| --- | --- | --- | --- |
| 0x0 | Identification String | “ABUM” – A/B Update Mechanism | 0x4D554241 |
| 0x4 | Version | 0x1 | 0x00000001 |
| 0x8 | Length | 4 – No of registers present (not including the first 4) | 0x00000004 |
| 0xC | Checksum | SUM of all words excluding checksum | 0xAEB1BDB9 |
| 0x10 | Persistent State register | See table below | 0x01010000 |
| 0x14 | Image A offset | CSU multi-boot address offset of "Image A". Must correspond to 32 KB boundary. | 0x00200000 |
| 0x18 | Image B offset | CSU multi-boot address offset of "Image B". Must correspond to 32 KB boundary | 0x00F80000 |
| 0x1C | Recovery offset | CSU multi-boot address offset of "Recovery Image". Must correspond to 32 KB boundary. | 0x01E00000 |

| 0x00100010 | 0x4 |
| --- | --- |

Persistent Register¶

| Bit width | Field | Type | Description |
| --- | --- | --- | --- |
| 7:0 | Last Image Booted | R | 0 - Image A, 1 - Image B |
| 15:8 | Requested Boot Image | RW | 0 - Image A, 1 - Image B |
| 23:16 | Image B Bootable | RW | 0 - Not bootable, 1 - Bootable |
| 31:24 | Image A Bootable | RW | 0 - Not bootable, 1 - Bootable |